Pi2_receiver.c

code:

#include <stdio.h>

#include <wiringPi.h>

const int ledPin   = 23; // BCM23 (physical 16)

const int sigInPin = 24; // BCM24 (physical 18)

int main(void)

{

if (wiringPiSetupGpio() == -1) return 1;

pinMode(ledPin, OUTPUT);

pinMode(sigInPin, INPUT);

pullUpDnControl(sigInPin, PUD_DOWN); // default low, stable

printf("Pi2 receiver running...\n");

int last = LOW;

while (1) {

int now = digitalRead(sigInPin);

// detect rising edge: 0 -> 1

if (last == LOW && now == HIGH) {

digitalWrite(ledPin, HIGH);

delay(2000);

digitalWrite(ledPin, LOW);

// wait until signal returns low to avoid retrigger

while (digitalRead(sigInPin) == HIGH) {

delay(1);

}

}

last = now;

delay(1);

}

return 0;

}

Pi1_sender.c

code:

#include <stdio.h>

#include <wiringPi.h>

const int ledPin    = 23; // BCM23 (physical 16)

const int butPin    = 17; // BCM17 (physical 11) active-low

const int sigOutPin = 24; // BCM24 (physical 18) -> Pi2

static void sendPulse(int ms)

{

digitalWrite(sigOutPin, HIGH);

delay(ms);

digitalWrite(sigOutPin, LOW);

}

int main(void)

{

if (wiringPiSetupGpio() == -1) return 1;

pinMode(ledPin, OUTPUT);

pinMode(sigOutPin, OUTPUT);

digitalWrite(sigOutPin, LOW);

pinMode(butPin, INPUT);

pullUpDnControl(butPin, PUD_UP); // released=1, pressed=0

printf("Pi1 sender running...\n");

int last = HIGH; // because pull-up, idle is HIGH

while (1) {

int now = digitalRead(butPin);

// falling edge: 1 -> 0 means "pressed"

if (last == HIGH && now == LOW) {

delay(30); // debounce

if (digitalRead(butPin) == LOW) { // still pressed

// Pi1 LED on for 2s

digitalWrite(ledPin, HIGH);

delay(2000);

digitalWrite(ledPin, LOW);

// send signal to Pi2

sendPulse(100); // 100ms pulse

// wait until button released (avoid repeats)

while (digitalRead(butPin) == LOW) {

delay(10);

}

}

}

last = now;

delay(1);

}

return 0;

}