#include <stdio.h>

#include <stdlib.h>

#include <wiringPi.h>

#include <wiringSerial.h>

#include <lcd.h>

int robot;

/* HC-SR04 (wiringPi pin numbers) */

#define TRIG_WPI 28

#define ECHO_WPI 29

/* LCD1602 (wiringPi pins) */

#define LCD_RS 3

#define LCD_E  0

#define LCD_D4 6

#define LCD_D5 1

#define LCD_D6 5

#define LCD_D7 4

/* 超声波测距 */

float getDistance()

{

long startTime, travelTime;

digitalWrite(TRIG_WPI, LOW);

delayMicroseconds(2);

digitalWrite(TRIG_WPI, HIGH);

delayMicroseconds(10);

digitalWrite(TRIG_WPI, LOW);

while(digitalRead(ECHO_WPI) == LOW);

startTime = micros();

while(digitalRead(ECHO_WPI) == HIGH);

travelTime = micros() - startTime;

float distance = travelTime / 58.0;

return distance;

}

/* 启动车（左轮更快） */

void startCar()

{

serialPrintf(robot,"#ha");

delay(100);

serialPrintf(robot,"#sa");

// Motor1 右前

serialPutchar(robot,40);

serialPutchar(robot,0);

// Motor2 右后

serialPutchar(robot,40);

serialPutchar(robot,0);

// Motor3 左前（更快）

serialPutchar(robot,45);

serialPutchar(robot,0);

// Motor4 左后（更快）

serialPutchar(robot,45);

serialPutchar(robot,0);

}

/* 停车 */

void stopCar()

{

serialPrintf(robot,"#sa");

for(int i=0;i<4;i++)

{

serialPutchar(robot,0);

serialPutchar(robot,0);

}

}

/* 后台播放音频 */

void playAudio()

{

system("mpg123 -q -o alsa -a hw:1,0 music1.mp3 &");

}

/* 停止音频 */

void stopAudio()

{

system("killall mpg123");

}

int main()

{

float distance;

wiringPiSetup();

robot = serialOpen("/dev/ttyS0",57600);

if(robot < 0)

{

printf("Serial open failed\n");

return 1;

}

pinMode(TRIG_WPI, OUTPUT);

pinMode(ECHO_WPI, INPUT);

int lcd;

lcd = lcdInit(2,16,4,

LCD_RS,

LCD_E,

LCD_D4,

LCD_D5,

LCD_D6,

LCD_D7,

0,0,0,0);

if(lcd < 0)

{

printf("LCD init failed\n");

return 1;

}

printf("System start\n");

lcdClear(lcd);

lcdPosition(lcd,0,0);

lcdPrintf(lcd,"Robot Start");

delay(2000);

startCar();

while(1)

{

distance = getDistance();

if(distance < 2 || distance > 400)

{

continue;

}

printf("Distance: %.2f cm\n",distance);

lcdClear(lcd);

lcdPosition(lcd,0,0);

lcdPrintf(lcd,"Dist: %.2f cm",distance);

if(distance <= 30)

{

stopCar();

lcdPosition(lcd,0,1);

lcdPrintf(lcd,"STOP");

delay(2000);

/* 播放5秒 */

lcdClear(lcd);

lcdPosition(lcd,0,0);

lcdPrintf(lcd,"State:");

lcdPosition(lcd,0,1);

lcdPrintf(lcd,"Play");

playAudio();

delay(5000);

stopAudio();

/* 等待5秒 */

lcdClear(lcd);

lcdPosition(lcd,0,0);

lcdPrintf(lcd,"State:");

lcdPosition(lcd,0,1);

lcdPrintf(lcd,"Delay");

delay(5000);

/* 再播放5秒 */

lcdClear(lcd);

lcdPosition(lcd,0,0);

lcdPrintf(lcd,"State:");

lcdPosition(lcd,0,1);

lcdPrintf(lcd,"Play");

playAudio();

delay(5000);

stopAudio();

break;

}

delay(200);

}

serialClose(robot);

return 0;

}